Project Report

Project Title: Local Intranet System for Student Services
Student Names: Ayoub Aghbar, Nsralla Dabeek
Student IDs: 12428124, 12428610
Course: Computer Networks
Instructor: Jihad M. Hamamreh

1. Introduction

This project focuses on designing and implementing a local network that provides essential services for students without requiring internet access. The system simulates a real-world campus network using OpenWrt as a router and Ubuntu as a server hosting multiple services.

The goal is to provide a secure, controlled, and efficient intranet environment.

2. Objectives

- Provide a centralized announcement system

- Enable structured assignment submission

- Implement a file repository for course materials

- Simulate real-world network routing and segmentation

- Integrate multiple services into a unified system

3. Network Design

The network consists of three main components:

- LAN (Students)

- Server Network

- Router (OpenWrt)

Topology:

Students → Switch (Access Point) → OpenWrt → Server

The switch is used to simulate a wireless AP for now until other methods are figured out.

4. Technologies Used

- OpenWrt (Router & Firewall)

- Ubuntu Server

- Flask (Web Application)

- Apache (Web Server)

- Samba (File Sharing)

- SQLite (Database)

5. Implemented Services

5.1 Announcement System (Flask)

- Students can view announcements

- Admin can publish and delete announcements

5.2 File Sharing (Samba)

- Shared folder accessible by all students

- Used to distribute lecture materials

5.3 Assignment Submission (Flask)

- Students upload assignments to the server

- Files stored locally

5.4 Offline Resources (Apache)

- Educational files hosted locally

- Accessible via browser

6. Firewall Configuration

- Zones created: LAN, Server, WAN

- Controlled traffic using forwarding rules

- Internet access enabled only when needed

- Secure communication enforced between networks

7. Challenges Faced

- Network routing issues

- Firewall misconfiguration

- DNS resolution problems

- Service access permissions

8. Solutions

- Corrected routing tables

- Adjusted firewall forwarding rules

- Configured DNS manually

- Fixed Samba authentication issues

9. Conclusion

The project successfully demonstrates a functional intranet system providing essential services to students. It shows how local networks can operate independently of the internet while maintaining usability and efficiency.